Programming Quantum user interface using microVision3 from Keil using ULINK2

A functioning board needs 2 firmware; boot loader and application firmware.

They are located in 2 different spaces in the micro flash.

The boot loader function is the following:

On power up the boot loader firmware looks for the SD card. If SD card is present, the boot loader looks for a file named (UIM.hex) this is the main APPLICATION firmware. If the file exists, the boot loader will download it to the flash, then delete the (UIM.hex) from the SD card, and start executing the main APPLICATION firmware. If the SD card is not present or UIM.hex doesn’t exist on the card, then the boot loader will look for any (old) application in the flash and execute it.

The APPLICATION firmware is the main code that runs all the user inter face functions.

When compiling and building using the above tools, it gives you the option to specify the output file name. It also gives you the option to produce (.hex file) in addition to the (.axf) map file.

When compiling the APPLICATION we need to produce the (.hex file) so we can update the application (field update) using SD card.

When compiling the BOOT LOADER we don’t need to produce the (.hex file) since boot loader can’t be downloaded using SD card.

Programming the board:

- Case 1: If brand-new board using ULINK2.

  - Download the boot loader. The file name is (UB112009.axf) or newer version.

  - Download the User Interface main APPLICATION. The file name is (UI082010.axf) or newer version.

- Case 2: If a board is previously programmed as in case 1 or programmed with the boot loader only:

  - You can use SD card to download/update the APPLICATION by copying the APPLICATION (.hex) file to SD card and renaming it (UIM.hex). For example if you want to update a board with (UI082010.hex) code, rename it (UIM.hex) and copy it to SD card. Note that after update is complete the (UIM.hex) will be automatically deleted form the SD card. To prevent this from happening, you can save a copy under different name on the SD card.