8-10-12

Problem:  During startup on bench scrub module and master aren’t starting at same together and scrub module thinks it is going to the stopped state because it isn’t seeing a master heartbeat.

Temporary Solution: Disabling monitoring of master heartbeat in scrub module, Scrub(8-10-12) Production P1 build configuration, until issue can be resolved.

Is system more robust if the master tells the scrub module when to start monitoring for the heartbeat?

This scheme would allow modules to automatically know how long to go before declaring a no master heartbeat condition.  It also allows control of the when the module starts monitoring the heartbeat.

What happens when module stops responding what does the master controller’s stack return?

Initially is returns a 1 which is a

NMS_k_HBEAT_WAITING_ON_FIRST_MESSAGE

Does a 1 mean different things?   Initially it appears to be 1 until a heartbeat is returned, later it appears to mean that the heartbeats from the scrub module have stopped being received.

Yes the 1 state being returned means no heartbeats yet as well as    heartbeats received and not they stopped.

The 1 means no heartbeats received as well as error receiving heartbeats.

We really don’t want to talk to scrub module unless the state is Operational or Pre-Operational.  So monitorNodes() needs to initiate the on-line processing when it sees that the module has entered pre-op or op for the first time. Two conditions here one is where communications hasn’t started which should be treated like a warning, and the other where there was communications started and stopped which is a fault.

How should the master deal with a module that hasn’t sent a heartbeat yet?

Should monitor nodes delay for a fixed period of time before acknowleging the 1 state as an error that needs to put the module offline? or is it better to ignore the 1 state until a non 1 state has been received?

SDO’s shouldn’t be sent to a module unless the modules state is preoperational or operational and the module’s error flag has not been set.

What is the best way to prevent SDO’s from going out if module isn’t in either the preoperational or operational state?

Method 1: Check in all the IL calls that issue SDO commands.

Method 2: Let the ICAN_Master stack make the check.

Scrub mode should only be entered if the module is operational.  If the module isn’t operational then PDO’s won’t be transmitted.  The module needs to be preoperational or operational for SDO’s to be processed.

Can the node error flag be renamed inhibit flag and then be used for error condition as well as the module not in op or preop?

Renamed node error flag to inhibit flag.

How many places are checking the module state via getNodeState?

DisplayNodeStatus(), helper routine for DISPLAY_PARAMETER stuff.

monitorNodes()

moduleOnline()

moduleOperational()

How often does monitorNode() get called?

IL_ScrubModuleOffline() is not the opposite of IL_ScrubModuleOnline(),

IL_ScrubModuleOffline() is used to determine if the heartbeats have started and then stopped or if the scrub module has put it’s self into the stopped state.

IL_ScrubModuleOnline() indicates that the node is in either the operational or preoperational state,

If the CAN_master is blocking SDO’s when inhibit flag is set then the interface layer doesn’t need to check for module being online before send SDO commands

Trying to scrub sets SCRUB_CAN_FAULT if module isn’t online.

Real code needs to declare an error and lock out the scrub node if the node isn’t ready by the time that normal mode starts operating.

Where does scrub node get initialized for normal mode?  CAN operation should be running be time through displaying the flash screen and determining what mode to run in.  If it isn’t running by then, we should declare a CAN fault which should result in needing to rekey machine.  Doing it this way would allow some wiggle room for not starting the modules at the exact instant the the master is started, which is what occurs on the bench.

Summarize why things ended up the way they did in regards to the whole CAN operation.  Is initial CAN fault check in the correct place?

Is heartbeat for scrub module being started too soon?