### Commercial Floor Care Equipment

#### 3530/3560/3580

#### Wet/Dry Vacuums

| ### Features Specifications |  |  |  |
| --- | --- | --- | --- |
| Product Name Model Number | 3580 607619 Pump-Out | 3560 607618 | 3530 607617 |
| Chassis/Body Construction Chassis Construction: Caster Diameter: Wheel Diameter: | Rotationally-molded polyethylene 4 in /10.16 cm 10 in /25.4 cm | Rotationally-molded polyethylene 4 in /10.16 cm 10 in /25.4 cm | Rotationally-molded polyethylene 4 in /10.16 cm 10 in /25.4 cm |
| Dimensions Product Height: Product Length: Product Weight: Product Width: | 35 in /88.9 cm 30 in /76.2 cm 64 lb /29.03 kg 20 in /50.8 cm | 35 in /88.9 cm 30 in /76.2 cm 63 lb /28.58 kg 20 in /50.8 cm | 35 in /88.9 cm 30 in /76.2 cm 63 lb /28.58 kg 20 in /50.8 cm |
| Dirt/Debris Recovery System Filter Description: | Includes one cloth & one paper bag | Includes one cloth & one paper bag | Includes one cloth & one paper bag |
| Power Cord Power Cord Length: | 50 ft /15.24 m | 50 ft /15.24 m | 50 ft /15.24 m |
| Productivity Maximum Coverage Rate Per Hour | 20,000 ft2/h /1858 m2/h | 20,000 ft2/h /1858 m2/h | 20,000 ft2/h /1858 m2/h |
| Vacuum System Airflow at Orifice: Vacuum Motor Amps: Vacuum Motor Diameter: Vacuum Motor Horsepower: Vacuum Motor Stages: Vacuum Motor Type: Vacuum Motor Voltage: Vacuum Motor Watts: Waterlift at Sealed Orifice: | 112 cfm /3.17 m3/h 10.7 A 5.7 in /14.48 cm 1.6 hp 2 stage Tangential discharge, incorporates float shut-off device 120 V 1253 watts 107 in /271.78 cm | 101 cfm /2.86 m3/h 10 A 5.7 in /14.48 cm 1.57 hp 3 stage Acustec bypass 120 V 1200 watts 114 in /289.56 cm | 103 cfm/2.91 m2/h 8.5 A 5.7 in /14.48 cm 1.3 hp 2 stage Acustec bypass 120 V 980 watts 92 in/ 233.68 cm |
| Recovery System Recovery Tank Wet Capacity Vacuum Hose Length: | 16 gal /60.57 L 10 ft /3.05 m | 16 gal /60.57 L 10 ft /3.05 m | 16 gal /60.57 L 10 ft /3.05 m |
| Accessory Tools Accessories Included: Optional Accessories | 10 in /25.4 cm x 1.5 in /3.81 cm vacuum hose and cloth filter bag 3580 Wet/Dry Vacuum: 39 in /99 cm, 33 in /84 cm and 26 in /66.04 cm front mount squeegee kits, wet recovery bag, 16 in /40.64 cm wet pick-up kit, contractor tool kit and 11in /27.94 cm dry tool kit | 10 in /25.4 cm x 1.5 in /3.81cm vacuum hose and cloth filter bag 3560 Wet/Dry Vacuum: 39 in /99 cm, 33 in /84 cm and 26 in /66.04 cm front mount squeegee kits, wet recovery bag, 16 in /40.64 cm wet pick-up kit, contractor tool kit and 11in /27.94 cm dry tool kit | 10 in /25.4 cm x 1.5 in /3.81cm vacuum hose and cloth filter bag 3530 Wet/Dry Vacuum 39 in /99 cm, 33 in /84 cm and 26 in /66.04 cm front mount squeegee kits, wet recovery bag, 16 in /40.64 cm wet pick-up kit, contractor tool kit and 11in /27.94 cm dry tool kit |
| Warranty | Tennant warrants the rotationally-molded polyethylene housings and parts on all of its machines to be free from defects in materials and workmanship, under normal use and service for ten (10) years to the original end user. Under this warranty, we guarantee the performance of non-polyethylene parts and components to be free from defects for up to three (3) years or 2,500 hours of operation, whichever comes first, to the original end user. Parts replaced or repaired under this warranty are guaranteed for the remainder of the original warranty period. Labor charges are covered for three (3) full years form the date of purchase for Tennant models, provided an authorized Tennant service provider is used for repair. |  |  |

3530/3560/3580